- Je vais continuer à faire fonctionner le GPS, et essayer de l’intégrer et le faire fonctionner peut-être avec le LIDAR,

- Tp utiliser le GPS pour se dépacer en exterieur ? lidar permet de détecter objet et GPS à les coordonnées

- Pour activer le GPS sur le robot il faut aller dans teltonika et l’enable :

- Le GPS se calibre seul lorsqu’il est activé, cependant il marche uniquement en extérieur.

- Le temps je reprends l’entrainement de mon ia sur le dataset, je refinetunerai sur les panneaux adaptés quand j’aurai le bon dataset :

nohup python3 -c "

from ultralytics import YOLO

model = YOLO('/home/user/dataset_bdd100k/runs/bdd100k_yolo11n_v2/weights/last.pt')

model.train(

data='/home/user/dataset_bdd100k/bdd100k.yaml',

epochs=30,

imgsz=640,

batch=8,

device='CPU',

workers=4,

patience=10,

fraction=0.4,

resume=True,

project='/home/user/dataset_bdd100k/runs',

name='bdd100k_yolo11n_v2'

)" > ~/dataset_bdd100k/training_v2.log 2>&1 &

tail -f ~/dataset_bdd100k/training_v2.log

GPS :

- Le topic du GPS est /fix du package ros2-ublox-zedf9p

- Ros2 topic echo /fix :

- header:

- stamp:

- sec: 1777880169

- nanosec: 30292763

- frame_id: gps

- status:

- status: -1

- service: 1

- latitude: 0.0

- longitude: 0.0

- altitude: 0.0

- position_covariance:

- - 18446744065119.617

- - 0.0

- - 0.0

- - 0.0

- - 18446744065119.617

- - 0.0

- - 0.0

- - 0.0

- - 14064098445420.135

- position_covariance_type: 2

- Le GPS renvoie une position fiable (quelques m). Je vais essayer de faire un tp qui utilise l’IMU, le GPS et Nav2. Je donne une position au robot et il utilise les capteurs et le GPS pour y aller (les capteurs servent uniquement à détecter les obstacles, c’est le GPS qui aura la position)

Voici l’architecture de l’algorithme. Le GPS à la position, l’IMU les orientations et roll,pitch et yaw.

Rtab ?

- L’odométrie de te dis de combien les roues ont tournées ! Si ca drift par exemple et que la position n’a pas changé le robot peut se perdre c’est pourquoi on la mets de paire avec l’IMU :

- Je vais créer un package pour utiliser le GPS de cette manière :

# Dans le conteneur

cd $ABSOLUTE_WORKSPACE/src

ros2 pkg create --build-type ament_python navigation_pkg \

--dependencies rclpy sensor_msgs nav_msgs geometry_msgs nav2_msgs action_msgs

Il faut creer les dossiers config et launch

On créer  ekf.yaml, Ce fichier configure la fusion GPS + IMU + odométrie :

ekf_filter_node:

ros__parameters:

frequency: 30.0

sensor_timeout: 0.1

two_d_mode: true

publish_tf: true

print_diagnostics: true

odom_frame: odom

base_link_frame: base_link

world_frame: odom

# Odométrie des roues

odom0: /odom

odom0_config: [true,  true,  false,

false, false, true,

false, false, false,

false, false, true,

false, false, false]

odom0_differential: false

odom0_relative: false

# IMU

imu0: /imu/data

imu0_config: [false, false, false,

false, false, false,   # ← orientation désactivée

false, false, false,

true,  true,  true,    # ← vitesses angulaires gardées

true,  true,  true]    # ← accélérations gardées

imu0_differential: false

imu0_relative: true                  # ← ajoute cette ligne

imu0_remove_gravitational_acceleration: true

Mtn le navsat.yaml, Convertit les coordonnées GPS en coordonnées cartésiennes :

navsat_transform_node:

ros__parameters:

frequency: 30.0

delay: 3.0

magnetic_declination_radians: 0.0349  # ~2° pour la France

yaw_offset: 0.0

zero_altitude: true

broadcast_cartesian_transform: true

broadcast_utm_transform_as_parent_frame: false

publish_filtered_gps: true

use_odometry_yaw: false

wait_for_datum: false

Maintenant le nav2_params.yaml :

bt_navigator:

ros__parameters:

global_frame: odom

robot_base_frame: base_link

odom_topic: /odometry/filtered

bt_loop_duration: 10

default_server_timeout: 20

navigators: ['navigate_to_pose', 'navigate_through_poses']

navigate_to_pose:

plugin: "nav2_bt_navigator::NavigateToPoseNavigator"

navigate_through_poses:

plugin: "nav2_bt_navigator::NavigateThroughPosesNavigator"

controller_server:

ros__parameters:

controller_frequency: 5.0

min_x_velocity_threshold: 0.001

min_y_velocity_threshold: 0.5

min_theta_velocity_threshold: 0.001

failure_tolerance: 0.3

progress_checker_plugins: ["progress_checker"]

goal_checker_plugins: ["general_goal_checker"]

controller_plugins: ["FollowPath"]

progress_checker:

plugin: "nav2_controller::SimpleProgressChecker"

required_movement_radius: 0.5

movement_time_allowance: 10.0

general_goal_checker:

stateful: true

plugin: "nav2_controller::SimpleGoalChecker"

xy_goal_tolerance: 0.5

yaw_goal_tolerance: 0.25

FollowPath:

plugin: "nav2_regulated_pure_pursuit_controller::RegulatedPurePursuitController"

desired_linear_vel: 0.3

lookahead_dist: 1.0

min_lookahead_dist: 0.5

max_lookahead_dist: 2.0

lookahead_time: 1.5

rotate_to_heading_angular_vel: 0.5

transform_tolerance: 0.1

use_velocity_scaled_lookahead_dist: false

min_approach_linear_velocity: 0.05

approach_velocity_scaling_dist: 1.0

use_collision_detection: true

max_allowed_time_to_collision_up_to_lookahead: 1.0

use_regulated_linear_velocity_scaling: true

use_fixed_curvature_lookahead: false

curvature_lookahead_dist: 1.0

use_cost_regulated_linear_velocity_scaling: false

regulated_linear_scaling_min_radius: 0.9

regulated_linear_scaling_min_speed: 0.25

use_rotate_to_heading: true

allow_reversing: false

rotate_to_heading_min_angle: 0.785

max_angular_accel: 3.2

max_robot_pose_search_dist: 10.0

local_costmap:

local_costmap:

ros__parameters:

update_frequency: 5.0

publish_frequency: 2.0

global_frame: odom

robot_base_frame: base_link

rolling_window: true

width: 10

height: 10

resolution: 0.1

robot_radius: 0.5

plugins: ["obstacle_layer", "inflation_layer"]

obstacle_layer:

plugin: "nav2_costmap_2d::ObstacleLayer"

enabled: true

observation_sources: lidar

lidar:

topic: /rslidar_points

data_type: "PointCloud2"

marking: true

clearing: true

max_obstacle_height: 2.0

min_obstacle_height: 0.1

obstacle_max_range: 10.0

obstacle_min_range: 0.0

raytrace_max_range: 12.0

raytrace_min_range: 0.0

inflation_layer:

plugin: "nav2_costmap_2d::InflationLayer"

cost_scaling_factor: 3.0

inflation_radius: 1.0

global_costmap:

global_costmap:

ros__parameters:

update_frequency: 1.0

publish_frequency: 1.0

global_frame: odom

robot_base_frame: base_link

robot_radius: 0.5

resolution: 0.1

rolling_window: true      # ← ajoute ça

width: 50                 # ← 50m x 50m

height: 50                # ← au lieu de la taille par défaut

track_unknown_space: true

plugins: ["obstacle_layer", "inflation_layer"]

obstacle_layer:

plugin: "nav2_costmap_2d::ObstacleLayer"

enabled: true

observation_sources: lidar

lidar:

topic: /rslidar_points

data_type: "PointCloud2"

marking: true

clearing: true

inflation_layer:

plugin: "nav2_costmap_2d::InflationLayer"

cost_scaling_factor: 3.0

inflation_radius: 1.0

planner_server:

ros__parameters:

expected_planner_frequency: 1.0

planner_plugins: ["GridBased"]

GridBased:

plugin: "nav2_navfn_planner/NavfnPlanner"

tolerance: 0.5

use_astar: false

allow_unknown: true

smoother_server:

ros__parameters:

smoother_plugins: ["simple_smoother"]

simple_smoother:

plugin: "nav2_smoother::SimpleSmoother"

tolerance: 1.0e-10

max_its: 1000

do_refinement: true

behavior_server:

ros__parameters:

costmap_topic: local_costmap/costmap_raw

footprint_topic: local_costmap/published_footprint

cycle_frequency: 10.0

behavior_plugins: ["spin", "backup", "drive_on_heading", "wait"]

spin:

plugin: "nav2_behaviors/Spin"

backup:

plugin: "nav2_behaviors/BackUp"

drive_on_heading:

plugin: "nav2_behaviors/DriveOnHeading"

wait:

plugin: "nav2_behaviors/Wait"

global_frame: odom

robot_base_frame: base_link

transform_timeout: 0.1

use_sim_time: false

simulate_ahead_time: 2.0

max_rotational_vel: 0.5

min_rotational_vel: 0.2

rotational_acc_lim: 3.2

waypoint_follower:

ros__parameters:

loop_rate: 20

stop_on_failure: false

waypoint_task_executor_plugin: "wait_at_waypoint"

wait_at_waypoint:

plugin: "nav2_waypoint_follower::WaitAtWaypoint"

enabled: true

waypoint_pause_duration: 0

velocity_smoother:

ros__parameters:

smoothing_frequency: 20.0

scale_velocities: false

feedback: "OPEN_LOOP"

max_velocity: [0.3, 0.0, 0.5]

min_velocity: [-0.3, 0.0, -0.5]

max_accel: [2.5, 0.0, 3.2]

max_decel: [-2.5, 0.0, -3.2]

odom_topic: /odometry/filtered

odom_duration: 0.1

deadband_velocity: [0.0, 0.0, 0.0]

velocity_timeout: 1.0

map_server:

ros__parameters:

yaml_filename: ""

map_saver:

ros__parameters:

save_map_timeout: 5.0

free_thresh_default: 0.25

occupied_thresh_default: 0.65

Le localization.launch.py (qui lancera la localization) :

import os

from launch import LaunchDescription

from launch_ros.actions import Node

from ament_index_python.packages import get_package_share_directory

def generate_launch_description():

pkg = get_package_share_directory('navigation_pkg')

ekf_node = Node(

package='robot_localization',

executable='ekf_node',

name='ekf_filter_node',

output='screen',

parameters=[os.path.join(pkg, 'config', 'ekf.yaml')],

remappings=[('odometry/filtered', 'odometry/filtered')]

)

navsat_node = Node(

package='robot_localization',

executable='navsat_transform_node',

name='navsat_transform_node',

output='screen',

parameters=[os.path.join(pkg, 'config', 'navsat.yaml')],

remappings=[

('imu/data',        '/imu/data'),

('gps/fix',         '/fix'),

('odometry/filtered', '/odometry/filtered'),

]

)

return LaunchDescription([

ekf_node,

navsat_node,

])

Le deuxième fichier de lancement, navigation.launch.py :

import os

from launch import LaunchDescription

from launch.actions import IncludeLaunchDescription

from launch.launch_description_sources import PythonLaunchDescriptionSource

from ament_index_python.packages import get_package_share_directory

def generate_launch_description():

pkg_nav2 = get_package_share_directory('nav2_bringup')

pkg      = get_package_share_directory('navigation_pkg')

nav2_launch = IncludeLaunchDescription(

PythonLaunchDescriptionSource(

os.path.join(pkg_nav2, 'launch', 'navigation_launch.py')

),

launch_arguments={

'params_file': os.path.join(pkg, 'config', 'nav2_params.yaml'),

'use_sim_time': 'false',

}.items()

)

return LaunchDescription([nav2_launch])le code qui va permettre au robot d’aller au point demandé, dans ce code on mets les waypoints auxquels ont veut que le robot aille :

import rclpy

from rclpy.node import Node

from rclpy.action import ActionClient

from nav2_msgs.action import FollowWaypoints

from geometry_msgs.msg import PoseStamped

from robot_localization.srv import FromLL

import math

class GPSWaypointFollower(Node):

def __init__(self):

super().__init__('gps_waypoint_follower')

# Client pour convertir GPS → coordonnées cartésiennes

self.ll_to_map = self.create_client(FromLL, '/fromLL')

# Client action Nav2

self.waypoint_client = ActionClient(

self, FollowWaypoints, 'follow_waypoints')

self.get_logger().info('GPSWaypointFollower prêt')

def lat_lon_to_pose(self, lat, lon, alt=0.0):

"""Convertit latitude/longitude en PoseStamped dans le repère map"""

while not self.ll_to_map.wait_for_service(timeout_sec=1.0):

self.get_logger().info('Attente service fromLL...')

from geographic_msgs.msg import GeoPoint

req = FromLL.Request()

req.ll_point = GeoPoint()

req.ll_point.latitude  = lat

req.ll_point.longitude = lon

req.ll_point.altitude  = alt

future = self.ll_to_map.call_async(req)

rclpy.spin_until_future_complete(self, future)

pose = PoseStamped()

pose.header.frame_id = 'map'

pose.header.stamp = self.get_clock().now().to_msg()

pose.pose.position.x = future.result().map_point.x

pose.pose.position.y = future.result().map_point.y

pose.pose.position.z = 0.0

pose.pose.orientation.w = 1.0

return pose

def follow_gps_waypoints(self, waypoints_gps):

"""

Envoie une liste de waypoints GPS au robot.

waypoints_gps : liste de tuples (latitude, longitude)

Exemple : [(48.8566, 2.3522), (48.8570, 2.3530)]

"""

self.get_logger().info(f'Conversion de {len(waypoints_gps)} waypoints GPS...')

poses = []

for lat, lon in waypoints_gps:

pose = self.lat_lon_to_pose(lat, lon)

poses.append(pose)

self.get_logger().info(f'Waypoint converti : lat={lat} lon={lon} '

f'→ x={pose.pose.position.x:.2f} '

f'y={pose.pose.position.y:.2f}')

self.get_logger().info('Envoi des waypoints à Nav2...')

while not self.waypoint_client.wait_for_server(timeout_sec=1.0):

self.get_logger().info('Attente Nav2...')

goal = FollowWaypoints.Goal()

goal.poses = poses

future = self.waypoint_client.send_goal_async(goal)

rclpy.spin_until_future_complete(self, future)

goal_handle = future.result()

if not goal_handle.accepted:

self.get_logger().error('Waypoints refusés par Nav2')

return

self.get_logger().info('Navigation démarrée !')

result_future = goal_handle.get_result_async()

rclpy.spin_until_future_complete(self, result_future)

self.get_logger().info('Navigation terminée !')

def main(args=None):

rclpy.init(args=args)

node = GPSWaypointFollower()

# ─── MODIFIE CES COORDONNÉES GPS ───────────────────────────────

# Mets ici les coordonnées GPS de tes waypoints

waypoints = [

(48.8566, 2.3522),   # Waypoint 1

(48.8570, 2.3530),   # Waypoint 2

(48.8575, 2.3515),   # Waypoint 3

]

# ────────────────────────────────────────────────────────────────

node.follow_gps_waypoints(waypoints)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

Maintenant le setup.py :

from setuptools import setup

import os

from glob import glob

package_name = 'navigation_pkg'

setup(

name=package_name,

version='0.0.1',

packages=[package_name],

data_files=[

('share/ament_index/resource_index/packages',

['resource/' + package_name]),

('share/' + package_name, ['package.xml']),

(os.path.join('share', package_name, 'config'),

glob('config/*.yaml')),

(os.path.join('share', package_name, 'launch'),

glob('launch/*.py')),

],

install_requires=['setuptools'],

zip_safe=True,

entry_points={

'console_scripts': [

'gps_waypoint_follower = navigation_pkg.gps_waypoint_follower:main',

],

},

)

Et finalement de package.xml :

<?xml version="1.0"?>

<package format="3">

<name>navigation_pkg</name>

<version>0.0.1</version>

<description>GPS navigation package</description>

<maintainer email="user@robot.com">user</maintainer>

<license>MIT</license>

<depend>rclpy</depend>

<depend>sensor_msgs</depend>

<depend>nav_msgs</depend>

<depend>geometry_msgs</depend>

<depend>nav2_msgs</depend>

<depend>robot_localization</depend>

<depend>geographic_msgs</depend>

<export>

<build_type>ament_python</build_type>

</export>

</package>

Pour le fonctionnement du GPS il y a différent statuts quand on fait un topic echo :

Statut -1 : le robot ne capte pas et le gps n’est pas fonctionnel

Statut 0 c’est localisation basique à quelques metres pres

Statut 2 c’est que le gps est en mode rtk et donc qu’il se localise parfaitement a qlq cm pres

Javais un probleme d’orientation dans le ekf donc j’utlise que le gyro et l’acc et plus l’imu

A partir de la j’ai un gps qui se repere à quelques metres prêt et un robot qui arrive à peu prêt à s’orienter, mais c’est vraiment trop peu précis. J’ai 5 décimales de la positions qui sont stables (qlq metres) et j’aimerai avoir qlq cm de précision donc il faut que j’utilise de rtk du gps.

J’ai installer rtklib qui fait les correction ntrip :

# Installe RTKLIB

sudo apt-get update && sudo apt-get install -y rtklib

PARLER LE RTK, CORRECTION ET DE CETTE LIB

Je créer un nœud ntrip_client.py :

import rclpy

from rclpy.node import Node

from rtcm_msgs.msg import Message as RtcmMessage

import socket

import base64

import threading

class NtripClient(Node):

def __init__(self):

super().__init__('ntrip_client')

# Paramètres Centipède

self.declare_parameter('host',       'caster.centipede.fr')

self.declare_parameter('port',       2101)

self.declare_parameter('mountpoint', 'RTCM3')

self.declare_parameter('username',   'centipede')

self.declare_parameter('password',   'centipede')

self.host       = self.get_parameter('host').value

self.port       = self.get_parameter('port').value

self.mountpoint = self.get_parameter('mountpoint').value

self.username   = self.get_parameter('username').value

self.password   = self.get_parameter('password').value

# Publisher corrections RTCM vers le récepteur GPS

self.pub_rtcm = self.create_publisher(

RtcmMessage, '/rtcm', 10)

self.connected = False

self.socket    = None

# Lance la connexion dans un thread séparé

self.thread = threading.Thread(target=self.connect_and_stream)

self.thread.daemon = True

self.thread.start()

self.get_logger().info(

f'NTRIP client démarré → {self.host}:{self.port}/{self.mountpoint}')

def connect_and_stream(self):

while rclpy.ok():

try:

self.get_logger().info('Connexion au caster Centipède...')

self.socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

self.socket.settimeout(10.0)

self.socket.connect((self.host, self.port))

# Requête HTTP NTRIP

credentials = base64.b64encode(

f'{self.username}:{self.password}'.encode()).decode()

request = (

f'GET /{self.mountpoint} HTTP/1.0\r\n'

f'Host: {self.host}\r\n'

f'Ntrip-Version: Ntrip/2.0\r\n'

f'User-Agent: NTRIP ROS2Client/1.0\r\n'

f'Authorization: Basic {credentials}\r\n'

f'\r\n'

)

self.socket.sendall(request.encode())

# Vérifie la réponse

response = self.socket.recv(1024).decode('latin-1')

if 'ICY 200 OK' in response or '200 OK' in response:

self.get_logger().info('Connecté à Centipède ✅')

self.connected = True

self.stream_rtcm()

else:

self.get_logger().error(f'Connexion refusée : {response[:100]}')

except Exception as e:

self.get_logger().error(f'Erreur NTRIP : {e}')

self.connected = False

# Attends 5 secondes avant de réessayer

import time

time.sleep(5.0)

def stream_rtcm(self):

"""Lit les corrections RTCM et les publie sur /rtcm"""

buffer = b''

while rclpy.ok() and self.connected:

try:

data = self.socket.recv(4096)

if not data:

self.get_logger().warn('Connexion NTRIP perdue')

self.connected = False

break

buffer += data

# Parse les messages RTCM3 (commencent par 0xD3)

while len(buffer) >= 3:

if buffer[0] != 0xD3:

buffer = buffer[1:]

continue

# Longueur du message RTCM

msg_len = ((buffer[1] & 0x03) << 8) | buffer[2]

total_len = msg_len + 6  # header + data + CRC

if len(buffer) < total_len:

break  # attend plus de données

rtcm_data = buffer[:total_len]

buffer = buffer[total_len:]

# Publie le message RTCM

msg = RtcmMessage()

msg.header.stamp = self.get_clock().now().to_msg()

msg.header.frame_id = 'gps'

msg.message = list(rtcm_data)

self.pub_rtcm.publish(msg)

except Exception as e:

self.get_logger().error(f'Erreur stream RTCM : {e}')

self.connected = False

break

def main(args=None):

rclpy.init(args=args)

node = NtripClient()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

'ntrip_client = navigation_pkg.ntrip_client:main', # ← ajoute ça dans le setup.py

Changer le localization.launch.py du coup :

import os

from launch import LaunchDescription

from launch_ros.actions import Node

from ament_index_python.packages import get_package_share_directory

def generate_launch_description():

pkg = get_package_share_directory('navigation_pkg')

ekf_node = Node(

package='robot_localization',

executable='ekf_node',

name='ekf_filter_node',

output='screen',

parameters=[os.path.join(pkg, 'config', 'ekf.yaml')],

remappings=[('odometry/filtered', 'odometry/filtered')]

)

navsat_node = Node(

package='robot_localization',

executable='navsat_transform_node',

name='navsat_transform_node',

output='screen',

parameters=[os.path.join(pkg, 'config', 'navsat.yaml')],

remappings=[

('imu/data',          '/imu/data'),

('gps/fix',           '/fix'),

('odometry/filtered', '/odometry/filtered'),

]

)

ntrip_node = Node(

package='navigation_pkg',

executable='ntrip_client',

name='ntrip_client',

output='screen',

parameters=[{

'host':       'caster.centipede.fr',

'port':       2101,

'mountpoint': 'RTCM3',  # ← change par ta station la plus proche

'username':   'centipede',

'password':   'centipede',

}]

)

return LaunchDescription([

ekf_node,

navsat_node,

ntrip_node,

])

J’avais des problèmes dans le .yaml, de plus le robot était considére comme base fixe et non base mobile donc j’ai change le zed_f9p.yaml (celui de ublox_gps) :

ublox_gps_node:

ros__parameters:

debug: 0

device: /dev/ttyACM0

frame_id: gps

uart1:

baudrate: 460800

tmode3: 0

rtcm:

ids: [0, 1, 2, 3, 4, 5]

rates: [1, 1, 1, 1, 1, 1]

publish:

all: false

rxm:

rtcm: true

nav:

all: true

relposned: true

posllh: true

posecef: true

inf:

all: true

config_on_startup: false    # ← ne pas reconfigurer au démarrage

# Sur l'hôte

sudo ip link set can0 up type can bitrate 500000 sudo ip link set can0 txqueuelen 1000

permet de mettre CAN0 à l’état UP

EXPLIQUER CAN0

La correction rtk ne veut pas fonctionner j’ai essayer cette solution :

Mais ca ne marche toujours pas.

EXPLIQUER CENTIPEDE

Le rtk ne fonctionne vraiment pas

Je vais essayer de remettre comme c’était avant et remettre le rover en mode statique pour essayer. Le robot a de base un topic rtcm donc je vais l’étudier et voir s’il fonctionne de base.